www.observator.com

Technical note # OMC-048 Scriptable data logger Config.txt and recovery.txt templates

Version: 20250930 Status: Released Confidentiality: Not confidential Date: 30 September 2025 Author: R. Beun

Preface

The OMC-048 data logger is configured by creating a config.txt file. To help you create a new config.txt file, this document contains a table with pieces of text that can be copied to the config.txt file. Once ready, the config.txt file must be copied to the FLASH of your OMC-048 logger.

In case you want to use ‘Blue2Cast Remote Control’, you also need a recovery.txt file. At the end of this document, a table is given for creating a new recovery.txt file. This file should also be placed on the FLASH.

For creating the config.txt file please use notepad or notepad++ or another suitable (ASCII) editor to copy the text to.

In the below tables, note that everything to the right of a ‘:’ is just an example value. You need to change that value for your situation. For example, in below tables it may read:

- id: yourchoice

port: 1

Then, obviously, you need to replace yourchoice and indicate the correct number of the port. So, for example, you may change it to:

- id: level_sens1

port: 2

Please refer to the ‘OMC-048 configuration manual’ for more information.

Beware that the confix.txt file is sensitive to (leading) spaces! Make sure that lines have 0 or 2 leading spaces. No tabs.

=========== This document belongs to FW_1.03B2803 ================

Table of contents

1	File header	4

2	System	5

3	Cellular modem	6

4	Iridium modem	7

5	Onboard (internal) sensor drivers	8

6	Analog (voltage/current) sensor drivers	9

7	Digital (pulse/switch) input drivers	10

8	Serial sensor drivers	11

9	Generic drivers	15

10	Power and relay port drivers	18

11	Common settings for drivers	19

12	Log_parameters	20

13	The recovery.txt file	21

# File header

It is recommended to always have these blocks at the top of your file, for your own convenience.

| Text blocks to copy to config.txt | Comments |
| --- | --- |
| # ******************************************************** # FILE INFORMATION # ******************************************************** # # Created by : <your name, company> # Date : <date of creation/release> # FW version : <e.g. FW_1.03B2803> # ========================================================= # # < Description> # ******************************************************** # CHANGE LOG # ******************************************************** # # Date of change: <date> # Changed by : <name, company> # Description: # < Reason of making change, who requested the change, and so on> # # ******************************************************** | This whole text block is just comments for your own benefit. The logger ignores all lines starting with ‘#’ Please replace all text between ‘<’ and ‘>’ with your own text. |

# System

| These blocks must be present in EVERY config file | Comments |
| --- | --- |
| # ======================================================== # --------------System settings--------------------------- # ======================================================== Omc048: usb_mode: repl system_id: Paris_Nord application: Turb_monitor file_log_level: warn repl_log_level: info utc_time_offset_hours: +1 utc_time_offset_minutes: +0 sensor_data_print: True self_test: True Data_file: - id: data create_interval: "2 10 * * *" | repl/storage/debug, but don't use 'debug' debug/info/warn/error/fatal debug/info/warn/error/fatal All sensor data is printed in the REPL ‘False’ to speed-up the start-up process. Opening/closing data files on the SD card. "second minute hour day week-day" |

# Cellular modem

| Copy to config.txt if you use the cellular modem | Comments |
| --- | --- |
| # ======================================================== # --------------Cellular Modem-Settings------------------- # ======================================================== Modem: id: modem port: modem sim_username: sim_usr sim_password: sim_pwd apn: sim_apn network_technology: GSM + LTE iot_technology: CAT-M1 cellular_diagnostics: True cron_offset: False Connection_settings: data_protocol: FTP ftp_url: ftp.omc-data-online.com ftp_port: 21 ftp_username: <username> ftp_password: <password> transmit_interval: "0 10 * * *" utc_time_sync: True utc_time_server: europe.pool.ntp.org utc_time_server_port: 123 webserver_access: False webserver_url: blue2cast.com webserver_port: 5000 webserver_password: <Contact Observator> | Get these details from your SIM card provider 'GSM + LTE' or 'GSM' or 'LTE' ‘CAT-M1’ or ‘CAT-M1 + NB-IoT’ or ‘NB-IoT’ For internet connection through the cellular modem. FTP/SSL/None (e.g. for OMC-DOL/Blue2Cast/Nothing) Only relevant if ‘data_protocol: FTP’ for FTP/SSL. Also used for webserver_acces, if selected. Sync to an NTP server. Alternatively, see 'GPS' Blue2Cast remote control. Leave as-is; Blue2Cast will fill-in the password. |

# Iridium modem

| Copy to config.txt if you use the external Iridium modem OMC-IEM. | Comments |
| --- | --- |
| # ======================================================== # --------------Iridium Modem-Settings-------------------- # ======================================================== # # IMPORTANT: This driver uses a REFERENCE TABLE. # so you MUST customize the iridium_reference_tables file ! Iridium_modem: id: iridium_modem port: serial4 supply_port: 4 Iridium: - id: data_iridium system_id: Z transmit_interval: "10 0 * * *" add_timeout_field: False Iridium_file: - id: iridium create_interval: "0 0 * * *" | Don’t forget this! Baudrate and so on are fixed by the driver. Select a small id (like one character) to save on data transport cost. "second minute hour day week-day" Opening/closing data files for transmission over Iridium |

# Onboard (internal) sensor drivers

| Copy to config.txt if needed | Comments |
| --- | --- |
| Onboard: - id: onboard sample_interval: "30 8,18,28,38,48,58 * * *" | The onboard sensors are 'Int. Temperature', 'Int. Humidity', 'Int. Coin cell voltage'. For models with SN>x0200 also 'Supply voltage' |
| Input_power_monitor: - id: power_mon hysteresis_high: 12.5 hysteresis_low: 11.5 sample_interval: "30 0 * * *" | The power monitor allows you to let the logger shut down gracefully below a certain low voltage and restart above a high voltage. Loggers with SN>48000200 only. |

# Analog (voltage/current) sensor drivers

| Copy to config.txt for each analog input driver you need | Comments |
| --- | --- |
| Analog_voltage: - id: AV1 port: 1 sample_interval: "0 0 * * *" min_in: 0 max_in: 24000 min_out: 0 max_out: 24 log_name: Ext_voltage log_unit: V log_tag: VOLT1 | port 1 = pin 18 with range 0-24V, port 2 = pin 19 with range 0-5V |
| Analog_current: - id: AC2 port: 2 sample_interval: "0 0 * * *" min_in: 0 max_in: 24 min_out: 0 max_out: 100 log_name: Current1 log_unit: "%" log_tag: CURR1 | port 1 to 4 correspond to pin 23 to 26 Special characters like % should be between quotes. |

# Digital (pulse/switch) input drivers

| Copy to config.txt for each digital input driver you need | Comments |
| --- | --- |
| rain_pulse: - id: cnt port: 1 mm_per_pulse: 0.2 reset_interval: None sample_interval: "0 0 * * *" | Driver for a tipping-bucket rain gauge Also suitable for counting pulses from other sensors. Port 1 to 2 correspond to pin 51 to 52 or eg. reset_interval: "0 0 0 * *" To reset the counter at midnight. To sample the counter every hour. |
| input_state: - id: switch1 port: 1 state_invert: False trigger_on_event: True sample_interval: "0,10,20,30,40,50 * * * *" | To sample the state of the input pins. Port 1 to 2 correspond to pin 51 to 52 Or: sample_interval: False |

# Serial sensor drivers

| Copy to config.txt for each serial driver you need | Comments |
| --- | --- |
| EXO: - id: EX1 port: serial1 mode: RS232 baudrate: 9600 sample_interval: "0 0,10,20,30,40,50 * * *" supply_port: 1 supply_port_always_on: False wiper_interval: "0, 0,20,40 * * *" delay_after_wipe: 25 | This driver is specific for Xylem EXO sondes, connected by RS232 via a DCP. Make sure the EXO and DCP are configured properly before using this driver. Must coincide with the sample time points, but there can be less. |
| manta: - id: Manta1 port: serial1 baudrate: 19200 sample_interval: "0 0,10,20,30,40,50 * * *" response_timeout: 60 supply_port: 1 supply_port_always_on: False wiper_interval: "0 0,20,40 * * *" delay_after_wipe: 10 | This driver is specific for Eureka Manta sondes, connected by RS232. Make sure the sonde is configured properly before using this driver. Must coincide with the sample time points, but there can be less. |
| NEP5000: - id: NEP2 port: serial2 mode: RS485 baudrate: 9600 sample_interval: "0 * * * *" supply_port: 2 supply_port_always_on: False | This driver is specific for the NEP-5000 turbidity sensor in streaming mode. Make sure the sensor is configured properly before using this driver. |
| navilock_md6: - id: gps port: serial2 mode: RS232 baudrate: 9600 sample_interval: "0 0 * * *" supply_port: 2 supply_port_always_on: False startup_time: 60 response_timeout: 300 utc_time_sync: True | This driver is specific for the OMC-ANT-G09 (Navilock MD6) GNSS (GPS) receiver. You can use Gps: as an alias for navilock_md6: Sync to the GPS clock. You can also sync to NTP. Don't enable both. |
| # IMPORTANT: This driver uses the GMX501 table in # serial_input_reference_tables! gmx_501: - id: gmx port: serial1 mode: RS422 baudrate: 19200 sample_interval: "* * * * *" supply_port: 1 supply_port_always_on: True response_timeout: 2 utc_time_sync: True | This driver is for the Maximet GMX501 with GPS from Gill. Don’t forget to check the GMX501 table against your Maximet settings. Sync to the GPS clock of the GMX. |
| nortek_cp: - id: ncp port: 3 baudrate: 19200 rxbuf: 1024 sample_interval: "15 0,30 * * *" response_timeout: 1860 | This driver is specific for Nortek Aquadopp and AWAC sensors. It supports only current profiles in binary mode. Check the size of the binary output file |
| # IMPORTANT: This driver uses the indicated table in # modbus_reference_tables! modbus_opus: - id: opus sample_interval: "0 0,30 * * *" port: serial1 baudrate: 9600 mode: RS485 rxbuf: 1024 protocol: RTU reference_table: TriOS_OPUS measurement_wait_time: 25 response_timeout: 120 | This driver is specific for the TriOS OPUS sensor. It uses the generic modbus driver ‘modbus’. Thus the reference table must be customized to fit the sensor settings! Check the size data transfer (set of registers) in bytes |
| modbus_spectrolyser: - id: s port: serial2 baudrate: 38400 mode: RS485 rxbuf: 1024 protocol: RTU relay_port: 1 relay_port_always_on: True startup_time: 70 sample_interval: "35 9,19,29,39,49,59 * * *" measurement_wait_time: 25 reference_table: P,A,B,C,D spect_sleep_duration: 555 # 600-25-10-10 = response_timeout: 10 | This driver is specific for the s::can Spectrolyser V3 sensor. It uses the generic modbus driver ‘modbus’. Thus the reference table ‘modbus_reference_tables.py’ must be customized to fit the sensor settings! |
| modbus_seametrics: - id: seam1 protocol: RTU register_print: True sample_interval: "0 * * * *" supply_port: 1 startup_time: 1 port: serial1 mode: RS485 baudrate: 9600 reference_table: PT12,CT2X | This driver is specific for the Seametrics smart sensors sensor. It uses the generic modbus driver ‘modbus’. Thus the reference table must be customized to fit the sensor settings! |
| modbus_wimo: - id: wimo address: 0x80 wiper_interval: "0 * * * *" sample_interval: "0,10,20,30,40,50 * * * *" port: serial1 mode: RS232 baudrate: 9600 protocol: RTU register_print: True supply_port: 1 supply_port_always_on: True | This driver is specific for the NKE WiMo multi-parameter sondes. It requires the presence of the “modbus_reference_tables.py” file in the script/modules folder! No table is needed. |

# Generic drivers

These blocks must only be present if you use the corresponding generic driver.

| Copy to config.txt for each generic driver you need | Comments |
| --- | --- |
| # This driver uses the serial_input_reference_tables.py file! generic_serial_input: - id: gsi1 port: serial1 mode: RS485 baudrate: 9600 reference_table: table_gsi sample_interval: "0 * * * *" supply_port: 3 supply_port_always_on: False startup_time: 10 response_timeout: 60 | You must customize this file! Check this table with your sensor output. "second minute hour day week-day" |
| # This driver uses the nmea_reference_tables.py file. generic_nmea: - id: gnmea port: nmea mode: RS422 baudrate: 4800 reference_table: VTG,GGA,RMC sample_interval: "0 * * * *" supply_port: 3 supply_port_always_on: False startup_time: 10 response_timeout: 60 | You must customize this file! The NMEA protocol (this driver) can be used on any of the serial ports, including the nmea port Check these table(s) with your sensor output. |
| # This driver uses the sdi12_reference_tables.py file. generic_sdi12: - id: sens1 port: sdi12 # For SDI-12, you must indicate 1200 Bd, 7 bits, parity 0 baudrate: 1200 bits: 7 parity: 0 reference_table: sdi_sens1 sample_interval: "0 * * * *" supply_port: 1 supply_port_always_on: False startup_time: 1 | You must customize this file! Must be these values! ,, ,, You must customize this table! |
| # This driver uses the modbus_reference_tables.py file. modbus: - id: modbus_1 protocol: RTU register_print: True port: serial1 mode: RS485 rxbuf: 1024 baudrate: 9600 reference_table: mt1,mt2 | You must customize this file! Change to ‘False’ once the configuration is redy for deployment Default no parity |
| # This driver uses the serial_output_reference_tables.py file. generic_serial_output: - id: gso port: serial4 mode: RS232 baudrate: 9600 reference_table: table_gso | You must customize this file! You must customize this table! |
| Data_serial_out: - id: data_serial create_interval: "0,10,20,30,40,50 * * * *" port: serial1 baudrate: 115200 mode: RS232 txbuf: 1024 reference_table: mt | The table is optional. If present, customize the ‘serial_output_reference_tables.py file’ |

# Power and relay port drivers

The power and relay ports are mostly controlled from sensor drivers. This is done using the “supply_port” or “relay_port” settings. However, if a port is not used by any of the sensors drivers, it can be controlled from below driver.

| Copy to config.txt for each driver you need | Comments |
| --- | --- |
| pulse_driver: - id: wiper sample_interval: "0,20,40 * * * *" supply_pulse_port: 3 relay_pulse_port: 2 pulse_duration: 5 normally_off: True | Generates a pulse of the indicated duration and polarity on a power and/or relay port. |

# Common settings for drivers

You can add a selection of below lines to any of the sensor (input) drivers. Don’t forget that these lines need 2 leading spaces!

| Text lines to use in any sensor driver | Comments |
| --- | --- |
| startup_time: 10 response_timeout: 60 cooldown_time: 0 sample_interval: "0 * * * *" cron_offset: 00:00:12 average_interval: "30 * * * *" moving_average_window: 16 minimum_required_samples: 12 supply_port: 1 supply_port_always_on: False relay_port: 2 relay_port_always_on: True | Timing ,, ,, ,, ,, Averaging. Note that averaging over many samples uses a lot of internal memory! ,, ,, Power/switching ,, ,, ,, |

You can add a selection of below lines to (almost) any of the drivers that use a serial port. Don’t forget that these lines need 2 leading spaces!

| Text lines to use in (input/output) drivers that use serial ports | Comments |
| --- | --- |
| port: serial1 mode: RS485 baudrate: 9600 bits: 8 parity: None stop: 1 rxbuf: 128 txbuf: 128 | serial1/serial/2/serial3/serial4/nmea/SDI_12 RS232/RS422/RS485 settings 7/8 0/1/None (0=even, 1 =odd, None = no parity) Number of stop bits A data burst from the sensor should fit in the RX buffer (number of bytes) For output drivers. |

# Log_parameters

This block must only be present if you want to include only a selection of the parameters from specific sensors.

| Text blocks to copy to config.txt | Comments |
| --- | --- |
| Log_parameters: gps: - Latitude - Longitude - Satellites modem: - Signal strength | id of the sensor of which only the listed parameters should be logged. You may use ‘sensor-data-print: True’ to find the names of the parameters. id of another sensor of which only the listed parameters should be logged. |

# The recovery.txt file

All previous tables are intended for the config.txt file. Below table is the complete recovery.txt file.

The recovery file should only be put on the FLASH drive if cellular communication is used with remote control (webserver_access: True in the config file).

| Complete recovery.txt file | Comments |
| --- | --- |
| # ----System---- # Omc048: usb_mode: repl system_id: RECOVERY # ----Modem-Settings---- # Modem: id: onboard_modem port: modem sim_username: sim_password: apn: # ---- External connection settings ---- # Connection_settings: webserver_access: True webserver_url: blue2cast.com webserver_port: 5000 webserver_password: <TO_BE_GENERATED> transmit_interval: "0 0,5,10,15,20,25,30,35,40,45,50,55 * * *" # ---- Power monitor ---- # Input_power_monitor: - id: power_mon hysteresis_high: 12.5 hysteresis_low: 11.5 sample_interval: "30 0 * * *" | Leave as-is Make sure you have the correct SIM details filled-in (see config.txt) Leave as-is Leave as-is Leave as-is Leave as-is (Blue2Cast will fill-in a value) Leave as-is, or choose other time points.. If the application is battery-powered, put this block in to prevent the batteries from being deep-discharged in case the logger remains in recovery mode for a long time. Remove the power monitor if the application is mains-powered. Adapt the thresholds for your specific case (e.g. battery type). |

www.observator.com